WINTAK MESHTASTIC PLUGIN

Requirements Document

Project: Echo6 TAK Infrastructure

Version 1.0  |  March 2026

Classification: UNCLASSIFIED

# 1. Executive Summary

This document defines the requirements for a native WinTAK plugin that provides full Meshtastic mesh network integration within the TAK ecosystem. The plugin replaces the current standalone Python gateway service with a single C# DLL that loads directly into WinTAK, providing native UI integration, per-channel management, rich node telemetry, and bidirectional mesh communication without external service dependencies.

# 2. Background & Problem Statement

## 2.1 Current State

A modified fork of brian7704/TAK_Meshtastic_Gateway provides basic Meshtastic-to-TAK bridging via a standalone Python service managed by NSSM on Windows. The gateway connects to a Meshtastic node over TCP and emits CoT events via UDP multicast to WinTAK.

### 2.2 Current Capabilities

- Position/PLI forwarding from mesh nodes to WinTAK map

- GeoChat text message relay (mesh to TAK, TAK to mesh)

- Telemetry CoT with sensor data in remarks text (battery, voltage, temperature, humidity, pressure)

- Node info processing and callsign resolution

- ATAK plugin message passthrough (uncompressed)

### 2.3 Current Limitations

- All mesh nodes render as identical teal dots with no shortnames visible on map

- All chat traffic arrives in a single undifferentiated window regardless of Meshtastic channel

- No channel selection or filtering — gateway receives from all channels, transmits only on channel 0

- No way to view telemetry or neighbor info by clicking on a node — data buried in remarks text

- No UI for selecting which channel outbound messages target

- Requires Python runtime, NSSM service wrapper, and separate process management

- No native WinTAK integration — cannot use contact panels, layer toggles, settings UI, or map drawing APIs

# 3. Project Goals

- Deliver a native WinTAK plugin (.dll) that requires no external services, runtimes, or process managers

- Provide per-channel awareness with visual differentiation of mesh nodes by channel membership

- Enable channel-based chat routing so Meshtastic channels map to TAK chat groups

- Surface telemetry and sensor data natively when interacting with node markers on the map

- Display neighbor topology information on node selection

- Allow the operator to select which Meshtastic channel outbound messages target

- Maintain full compatibility with TAK Server 5.x CoT schema and WinTAK 4.x plugin SDK

# 4. Architecture

## 4.1 Approach: Pure C# Plugin

The plugin will be implemented as a pure C# DLL targeting the WinTAK plugin SDK. Meshtastic protobuf handling will be implemented natively in C# using generated classes from Meshtastic’s .proto definitions via the Google.Protobuf NuGet package. No Python subprocess, no IPC, no external service.

## 4.2 Key Dependencies

| Dependency | Purpose | Source |
| --- | --- | --- |
| WinTAK Plugin SDK | Plugin host, CoT dispatch, UI integration | tak.gov |
| Google.Protobuf | Protobuf serialization/deserialization | NuGet |
| Meshtastic .proto files | C# class generation for mesh packet types | github.com/meshtastic/protobufs |
| System.Net.Sockets | TCP connection to Meshtastic node | .NET Framework |

## 4.3 Communication Flow

WinTAK Plugin (.dll) ↔ TCP ↔ Meshtastic Node (e.g., AIDA-N2) ↔ RF Mesh Network

Inbound: Meshtastic packets arrive via TCP, are deserialized from protobuf, converted to CoT XML, and injected into WinTAK via CotDispatcher.Dispatch(). Outbound: User actions in WinTAK (chat, PLI) are captured by plugin event handlers, serialized to Meshtastic protobuf, and transmitted via TCP to the connected node.

# 5. Functional Requirements

## 5.1 Connection Management

| ID | Requirement | Priority |
| --- | --- | --- |
| CON-01 | Plugin connects to a Meshtastic node via TCP (hostname:port) configured in WinTAK settings panel | Must |
| CON-02 | Connection status indicator visible in plugin UI (connected/disconnected/reconnecting) | Must |
| CON-03 | Automatic reconnection with configurable retry interval on connection loss | Must |
| CON-04 | Support for multiple simultaneous Meshtastic node connections (future consideration) | Want |

## 5.2 Node Display & Identification

| ID | Requirement | Priority |
| --- | --- | --- |
| NOD-01 | Mesh nodes display on map with their shortname as the callsign label | Must |
| NOD-02 | Nodes are color-coded by Meshtastic channel membership using TAK team colors | Must |
| NOD-03 | Node icons distinguish device role (router, client, tracker, etc.) where available | Should |
| NOD-04 | Stale nodes (no position update within configurable timeout) visually dim or change icon | Should |
| NOD-05 | Node tooltip or detail panel shows: long name, short name, hardware model, firmware version, last heard timestamp | Must |

## 5.3 Channel Management

| ID | Requirement | Priority |
| --- | --- | --- |
| CHN-01 | Plugin discovers and lists all channels configured on the connected Meshtastic node | Must |
| CHN-02 | Each Meshtastic channel maps to a TAK contact group for chat routing | Must |
| CHN-03 | Operator can toggle individual channels on/off for receive filtering | Must |
| CHN-04 | Operator can select target channel for outbound chat messages via dropdown or selector | Must |
| CHN-05 | Channel-to-TAK-team-color mapping is configurable in settings | Should |
| CHN-06 | Default channel (index 0) is pre-selected for outbound unless operator changes it | Must |

## 5.4 Chat & Messaging

| ID | Requirement | Priority |
| --- | --- | --- |
| MSG-01 | Inbound mesh text messages appear in WinTAK chat, attributed to sending node’s shortname | Must |
| MSG-02 | Messages are routed to channel-specific chat groups, not a single undifferentiated window | Must |
| MSG-03 | Operator can compose and send messages to a selected Meshtastic channel from WinTAK chat | Must |
| MSG-04 | Direct messages (DM) to specific mesh nodes supported | Should |
| MSG-05 | ATAK plugin compressed messages (unishox2) decoded when possible, graceful fallback otherwise | Should |

## 5.5 Telemetry & Sensor Data

| ID | Requirement | Priority |
| --- | --- | --- |
| TEL-01 | Clicking a mesh node displays telemetry data: battery %, voltage, uptime, channel utilization | Must |
| TEL-02 | Environment sensor data displayed: temperature, relative humidity, barometric pressure, gas resistance, IAQ | Must |
| TEL-03 | Telemetry data shown in a structured detail panel, not just raw remarks text | Must |
| TEL-04 | Telemetry values update in near-real-time as new packets arrive | Should |
| TEL-05 | Historical telemetry graphing (e.g., battery over time, temperature trend) | Want |
| TEL-06 | Power metrics (voltage, current) displayed where available | Should |

## 5.6 Neighbor Topology

| ID | Requirement | Priority |
| --- | --- | --- |
| NBR-01 | Clicking a mesh node shows its neighbor list with SNR/RSSI values for each neighbor | Must |
| NBR-02 | Optional map overlay draws lines between nodes and their neighbors | Should |
| NBR-03 | Neighbor lines color-coded by signal quality (green = good, yellow = marginal, red = poor) | Should |
| NBR-04 | Topology overlay is toggleable via WinTAK layer control or plugin UI | Should |
| NBR-05 | Neighbor data refreshes as NEIGHBORINFO packets arrive | Must |

## 5.7 Additional Portnums (Future)

| ID | Requirement | Priority |
| --- | --- | --- |
| PRT-01 | WAYPOINT_APP: Mesh waypoints rendered as CoT markers on WinTAK map | Should |
| PRT-02 | DETECTION_SENSOR_APP: Detection events rendered as alert CoT | Want |
| PRT-03 | ALERT_APP: Mesh alerts forwarded as GeoChat alerts | Want |
| PRT-04 | PAXCOUNTER_APP: People counter data surfaced in node telemetry panel | Want |
| PRT-05 | TRACEROUTE_APP: Route visualization between nodes | Want |

# 6. Non-Functional Requirements

## 6.1 Performance

- Plugin must not degrade WinTAK UI responsiveness. All Meshtastic I/O on background threads.

- CoT injection rate capped to prevent flooding WinTAK event bus (configurable, default 1 event/second/node).

- Memory footprint for node state tracking should remain under 50MB for 200+ tracked nodes.

## 6.2 Reliability

- TCP connection loss must not crash WinTAK. Plugin degrades gracefully with reconnection attempts.

- Malformed Meshtastic packets logged and discarded, never propagated as invalid CoT.

- Plugin state persists across WinTAK restarts (connection settings, channel config, UI preferences).

## 6.3 Deployment

- Single .dll file (plus dependencies) loadable via WinTAK plugin mechanism.

- No external Python runtime, NSSM, or separate Windows services required.

- Configuration via WinTAK native settings panel.

- Target framework compatible with WinTAK’s .NET version (to be confirmed from SDK).

# 7. UI Requirements

## 7.1 Settings Panel

- Meshtastic node connection: hostname/IP, port, connection toggle

- Channel configuration: list of discovered channels with enable/disable toggles and team color assignment

- Telemetry display options: units (C/F, hPa/inHg), stale timeout, update interval

- Topology overlay toggle and SNR color thresholds

## 7.2 Node Detail View

When an operator taps/clicks a mesh node marker on the map, the detail panel should display:

- Node identity: long name, short name, hardware model, firmware version, mesh node number

- Telemetry section: battery %, voltage, uptime, channel utilization, air utilization

- Environment section: temperature (C and F), humidity, barometric pressure, gas resistance, IAQ

- Neighbor section: list of known neighbors with node name, SNR, and last update time

- Channel membership: which channels this node has been heard on

## 7.3 Channel/Chat Integration

- Each Meshtastic channel appears as a selectable chat group in WinTAK’s chat interface

- Outbound channel selector: dropdown or toggle visible when composing messages

- Unread message indicators per channel

# 8. Open Questions & SDK Dependencies

The following items require investigation of the WinTAK Plugin SDK before implementation can begin:

| Item | Question | Impact |
| --- | --- | --- |
| SDK Availability | Is the WinTAK Plugin SDK available for download from tak.gov? What version? | Blocks all development |
| .NET Target | What .NET Framework / .NET version does WinTAK host? (.NET Framework 4.x vs .NET 6+) | Determines project structure and NuGet compatibility |
| CotDispatcher API | How are CoT events injected programmatically? CotDispatcher.Dispatch() or alternate mechanism? | Core plugin functionality |
| Contact Panel API | Can plugins add custom sections to the contact/node detail panel? | Telemetry and neighbor display (TEL-03, NBR-01) |
| Map Drawing API | Can plugins draw freeform lines/shapes on the map programmatically? | Neighbor topology overlay (NBR-02, NBR-03) |
| Chat Group API | Can plugins create and manage chat groups/rooms programmatically? | Channel-to-group mapping (CHN-02) |
| Settings Integration | How do plugins register settings panels in WinTAK preferences? | Connection and config UI |
| Layer Control | Can plugins register toggleable layers in WinTAK’s layer manager? | Topology overlay toggle (NBR-04) |
| Plugin Lifecycle | What hooks exist for plugin init, shutdown, and WinTAK state changes? | Connection management, cleanup |

# 9. Phased Delivery Plan

## 9.1 Phase 1: Core Bridge

Minimum viable plugin that replaces the Python gateway with native WinTAK integration.

- TCP connection to Meshtastic node with protobuf deserialization

- Position/PLI injection via CotDispatcher with shortname callsigns

- Channel-aware team color assignment on node markers

- Basic chat relay with channel-to-group routing

- Settings panel for connection configuration

- Outbound channel selection for messages

## 9.2 Phase 2: Rich Telemetry & Topology

Structured data display and mesh topology visualization.

- Telemetry detail panel on node click (battery, voltage, environment sensors)

- Neighbor info display on node click with SNR values

- Topology line overlay between neighbor nodes

- Stale node indication and automatic cleanup

- Node role icons (router, client, tracker)

## 9.3 Phase 3: Extended Portnums & Polish

Additional Meshtastic application support and UX refinement.

- Waypoint, detection sensor, alert, and paxcounter portnum handling

- Traceroute visualization

- Historical telemetry trends

- Multi-node connection support

- ATAK plugin compressed message handling (unishox2 equivalent in C#)

# 10. Reference: Meshtastic Portnums

Portnum coverage matrix for the plugin. Priority reflects the phased delivery plan.

| Portnum | Description | Phase | Priority |
| --- | --- | --- | --- |
| POSITION_APP (3) | Node GPS position | 1 | Must |
| NODEINFO_APP (4) | Device info, shortname, longname, hardware | 1 | Must |
| TEXT_MESSAGE_APP (1) | Plain text messages | 1 | Must |
| TELEMETRY_APP (67) | Battery, environment, power, air quality sensors | 2 | Must |
| NEIGHBORINFO_APP (71) | Neighbor node list with SNR | 2 | Must |
| MAP_REPORT_APP (73) | Condensed position + node info | 1 | Should |
| ATAK_PLUGIN (72) | TAKPacket protobuf (PLI, chat from ATAK mesh users) | 1 | Should |
| WAYPOINT_APP (8) | Shared waypoints/markers | 3 | Should |
| TRACEROUTE_APP (70) | Hop-by-hop route data | 3 | Want |
| DETECTION_SENSOR_APP (68) | Detection/motion sensor events | 3 | Want |
| ALERT_APP (226) | Emergency/alert broadcasts | 3 | Want |
| PAXCOUNTER_APP (69) | People counter data | 3 | Want |

# 11. Glossary

| Term | Definition |
| --- | --- |
| CoT | Cursor on Target — XML schema used by TAK for exchanging geospatial event data |
| CotDispatcher | WinTAK API for programmatically injecting CoT events into the TAK event bus |
| DLL | Dynamic Link Library — compiled .NET assembly loaded by WinTAK as a plugin |
| IAQ | Indoor Air Quality index reported by BME680/BME688 environment sensors |
| Meshtastic | Open-source LoRa mesh networking firmware for low-power long-range communication |
| NSSM | Non-Sucking Service Manager — Windows service wrapper (current gateway deployment) |
| PLI | Position Location Information — CoT event type for tracking entity positions |
| Portnum | Meshtastic application port number identifying the payload type in a mesh packet |
| Protobuf | Protocol Buffers — Google’s binary serialization format used by Meshtastic |
| SNR | Signal-to-Noise Ratio — RF signal quality metric in dB |
| TAK | Team Awareness Kit — geospatial situational awareness platform (ATAK/WinTAK/iTAK) |
| WinTAK | Windows desktop client for the TAK ecosystem |